Q1. Explain the different approaches of Target Tracking in Sensor Networks with diagrams. (5 marks)

English:
Target tracking in WSNs can be formulated in three ways:

- Push-based formulation: Nodes compute the position of the target themselves and periodically notify (push) the data to the sink node. A cluster structure is commonly used here.

- Poll-based formulation: Nodes only register the presence of a target, permitting a low-cost query. Data reports are sent to the sink only when a query is raised. A tree structure is often used.

- Guided formulation: Some special nodes (beacon nodes) define a trajectory for the target. The tracker (mobile agent) follows this trail to intercept the target. A face structure is typically used.

The full target tracking process also has 6 steps: Target Detection → Node Cooperation → Position Computation → Prediction → Energy Management → Recovery (cyclic process).

Bangla:
WSN-এ target tracking তিনভাবে করা যায়:

- Push-based: নোডগুলো নিজেরাই target-এর position calculate করে এবং periodically sink node-কে push করে জানায়। এখানে সাধারণত cluster structure ব্যবহার হয়।

- Poll-based: নোডগুলো শুধু target-এর presence register করে রাখে (low-cost query permit করার জন্য)। Query আসলে তখনই data sink-এ পাঠানো হয়। এখানে tree structure ব্যবহার হয়।

- Guided: কিছু নির্দিষ্ট beacon node target-এর জন্য একটা trajectory ঠিক করে দেয়, আর tracker সেই trail follow করে target intercept করে। এখানে face structure ব্যবহার হয়।

পুরো target tracking process এ 6টা ধাপ থাকে: Target Detection → Node Cooperation → Position Computation → Prediction → Energy Management → Recovery (cyclically চলতে থাকে)।

Q2. Describe the architecture of AID (Agricultural Intrusion Detection) prototype using WSN. (5 marks)

English:
AID is a WSN-based prototype for agricultural intrusion detection with a 4-layer architecture:

- Layer 1 (Sensing): Uses two sensors — Passive Infrared (PIR) sensor detects the intruding object entering the field boundary, and an Ultrasonic sensor senses the distance at which the object is located.

- Layer 2 (Processing): A micro-controller processes the sensed data.

- Layer 3 (Wireless Routing): Data is routed wirelessly from sensor nodes through the gateway to the sink.

- Layer 4 (SMS and Alert): Upon intrusion detection, the sink communicates via GSM to send an SMS alert to the farmer's cell phone/house.

Bangla:
AID হলো WSN-ভিত্তিক একটা prototype যা কৃষিক্ষেত্রে অনুপ্রবেশ (intrusion) শনাক্ত করে, এর 4-layer architecture আছে:

- Layer 1 (Sensing): দুইটা sensor থাকে — PIR sensor field-এর boundary দিয়ে intruder ঢুকলে সেটা detect করে, আর Ultrasonic sensor সেই object কত দূরে আছে সেটা measure করে।

- Layer 2 (Processing): Micro-controller sense করা data process করে।

- Layer 3 (Wireless Routing): Sensor node থেকে gateway হয়ে sink-এ wirelessly data route হয়।

- Layer 4 (SMS and Alert): Intrusion detect হলে sink GSM এর মাধ্যমে কৃষকের cell phone-এ SMS alert পাঠায়।

Q3. What is WMSN? Explain topology management in WMSNs. (5 marks)

English:
Wireless Multimedia Sensor Networks (WMSNs) incorporate low-cost cameras (typically CMOS) into wireless sensor nodes.

- Camera Sensor (CS) nodes: Capture multimedia data (video, audio, scalar), expensive, resource-hungry, with directional sensing range.

- Scalar Sensor (SS) nodes: Sense scalar data (temperature, light, vibration), low-cost, omni-directional sensing range.

WMSNs consist of fewer CS nodes and many SS nodes. In applications like security surveillance and habitat monitoring, SS nodes cannot give precise information, so CS nodes are used instead.

Topology Management: Since video data is large (requires high bandwidth and consumes more battery), coverage (fast response to events) and connectivity (reliable video transfer to control center) must both be maintained. Misra et al. proposed a distributed topology management scheme using a Coalition Formation Game between CS and SS nodes to achieve coverage while balancing connectivity and network lifetime.

Bangla:
Wireless Multimedia Sensor Networks (WMSN) এ wireless sensor node-এর সাথে low-cost camera (সাধারণত CMOS) যুক্ত থাকে।

- Camera Sensor (CS) node: ভিডিও, অডিও, স্কেলার ডেটা capture করে, খরচ বেশি, resource বেশি লাগে, directional sensing range।

- Scalar Sensor (SS) node: তাপমাত্রা, আলো, ভাইব্রেশন এর মতো স্কেলার ডেটা সেন্স করে, কম খরচ, omni-directional sensing range।

WMSN এ CS node কম আর SS node বেশি থাকে। Security surveillance বা habitat monitoring এ SS node দিয়ে precise info পাওয়া যায় না, তখন CS node ব্যবহার করা হয়।

Topology Management: ভিডিও ডেটার সাইজ বড় (বেশি bandwidth ও battery লাগে) বলে coverage (event দ্রুত detect করা) এবং connectivity (video data control center এ পাঠানো) দুইটাই দরকার। Misra et al. একটা distributed topology management scheme দেন যেখানে CS ও SS node এর মধ্যে Coalition Formation Game ব্যবহার করে coverage, connectivity এবং network lifetime — তিনটাই balance করা হয়।

Q4. Explain Nanonetworks and the modes of communication among nanodevices. (5 marks)

English:
Nanodevices have components sized in nanometers. There are two main communication options among nanodevices:

- Electromagnetic-based Communication: Surface Plasmonic Polarition (SPP) is generated upon an electromagnetic beam, allowing propagation through graphene-based nano-antennas. EM communication for nanonetworks centers around the 0.1–10 Terahertz channel.

- Molecular Communication: Molecules are used as the information carrier. Information is packed into vesicles, and a gap junction acts as a mediator between cells and vesicles for information exchange between communicating entities. Research on this was performed at NTT, Japan lab.

Bangla:
Nanodevice-এর component গুলো nano-meter সাইজের হয়। Nanodevice-দের মধ্যে যোগাযোগের দুইটা প্রধান উপায় আছে:

- Electromagnetic-based Communication: Electromagnetic beam এর উপর Surface Plasmonic Polarition (SPP) generate হয়, যা graphene-based nano-antenna এর মাধ্যমে propagate করে। Nanonetworks-এর জন্য EM communication মূলত 0.1–10 Terahertz channel এ হয়।

- Molecular Communication: এখানে molecule কে information হিসেবে ব্যবহার করা হয়। Information vesicles এর ভেতর প্যাক করা থাকে, এবং gap junction, cell আর vesicle এর মধ্যে mediator হিসেবে কাজ করে information exchange করতে। এই গবেষণা NTT, Japan lab এ করা হয়েছিল।

Q5. Discuss the challenges related to Underwater Acoustic Sensor Networks (UWASNs). (5 marks)

English:
In a layered shallow oceanic region, non-linear internal waves called solitons are one of the major scatterers of underwater sound, and their effect on network performance is important to consider. If sensor nodes are deployed in such an environment, inter-node communication gets affected due to interaction of the wireless acoustic signal with solitons, greatly affecting network performance.

Existing performance-analysis works on UWASNs often lack consideration of major dominating oceanic forces that impart node mobility, and are typically limited to shallow depths/coastal areas. To address this, Mandal et al. proposed a physical mobility model called the Oceanic Forces Mobility Model (OFMM), which incorporates realistic oceanic forces so that nodes move in a 3D ocean column, giving more meaningful performance insights.

Bangla:
একটা layered shallow oceanic region এ non-linear internal waves, যাদের soliton বলা হয়, সেগুলো underwater sound এর major scatterer, তাই network performance এ এর প্রভাব consider করা জরুরি। এমন environment এ sensor node deploy করা হলে wireless acoustic signal সোলিটনের সাথে interact করে যার ফলে inter-node communication এবং network performance ব্যাপকভাবে affected হয়।

UWASN নিয়ে বিদ্যমান কাজগুলোতে সাধারণত node mobility-র জন্য দায়ী major oceanic force গুলো consider করা হয় না, এবং সেগুলো শুধু shallow depths/coastal area তে সীমাবদ্ধ থাকে। এই সমস্যা সমাধানের জন্য Mandal et al. একটা physical mobility model প্রস্তাব করেন — Oceanic Forces Mobility Model (OFMM) — যেখানে বাস্তব oceanic force গুলোকে বিবেচনা করে node গুলোকে 3D ocean column-এ movement করানো হয়, যাতে বেশি meaningful performance insight পাওয়া যায়।

Q6. Explain HASL: High-Speed AUV-Based Silent Localization for Underwater Sensor Networks. (5 marks)

English:
HASL is a silent and energy-efficient localization scheme for mobile Underwater Sensor Networks (UWSNs), performed in three phases:

- Beacon Sending: The AUV gets its GPS location, starts dead-reckoning, and broadcasts a beacon at constant intervals.

- Beacon Reception: Underwater nodes perform silent listening (no need to transmit, saving energy) and receive an "effective" set of beacon messages.

- Location Estimation: Using trilateration along with depth (z) obtained from a pressure sensor, the node estimates its own location.

This scheme requires only 3 surface anchor nodes and uses mobility prediction for enhanced accuracy, along with an iterative approach requiring fewer initiator/anchor nodes.

Bangla:
HASL হলো mobile Underwater Sensor Network (UWSN) এর জন্য একটা silent ও energy-efficient localization scheme, যেটা তিনটা ধাপে কাজ করে:

- Beacon Sending: AUV নিজের GPS location নেয়, dead-reckoning শুরু করে, এবং নির্দিষ্ট বিরতিতে beacon broadcast করে।

- Beacon Reception: Underwater node গুলো silent listening করে (transmit করতে হয় না বলে energy বাঁচে) এবং একটা "effective" set of beacon message receive করে।

- Location Estimation: Trilateration এবং pressure sensor থেকে পাওয়া depth (z) ব্যবহার করে node নিজের location estimate করে।

এই scheme-এ মাত্র 3টা surface anchor node লাগে এবং accuracy বাড়ানোর জন্য mobility prediction ব্যবহার করা হয়, সাথে iterative approach থাকায় কম initiator/anchor node দরকার হয়।

Q7. What is Opportunistic Localization? Explain with the perspective of localized and unlocalized nodes. (5 marks)

English:
Opportunistic localization is a game-theoretic topology control approach for localization in sparse underwater sensor networks, proposed by Misra, Ojha, and Mondal. It has two objectives depending on node type:

- Unlocalized nodes: Aim to get localized with minimum localization delay.

- Localized nodes: Select a transmission power level such that the maximum number of neighboring nodes can be localized with minimum energy consumption.

From an unlocalized node's perspective, it looks at its neighbours (localized/unlocalized) within its sensing range to opportunistically get localized. From a localized node's perspective, it considers other nodes within its Opportunistic Power Range (OPR) to help localize them efficiently.

Bangla:
Opportunistic Localization হলো Misra, Ojha, ও Mondal এর প্রস্তাবিত একটা game-theoretic topology control approach, যেটা sparse underwater sensor network এ localization এর জন্য ব্যবহৃত হয়। এর দুইটা objective node type অনুযায়ী আলাদা:

- Unlocalized node: যত কম delay-তে সম্ভব localize হওয়া তার লক্ষ্য।

- Localized node: এমন একটা transmission power level select করে যাতে minimum energy খরচ করে maximum সংখ্যক neighbour node-কে localize করা যায়।

Unlocalized node এর perspective থেকে, সে তার sensing range এর মধ্যে থাকা neighbour (localized/unlocalized) দেখে opportunistically localize হওয়ার চেষ্টা করে। Localized node এর perspective থেকে, সে তার Opportunistic Power Range (OPR)-এর মধ্যে থাকা অন্য node গুলোকে efficiently localize করতে সাহায্য করে।

Q8. Explain Tic-Tac-Toe-Arch: A Self-Organizing Virtual Architecture for Underwater Sensor Networks along with Virtual Topology Formation process. (5 marks)

English:
Tic-tac-toe-arch is a self-organizing virtual architecture for underwater sensor networks that (1) calculates the duration of connectivity between underwater nodes and (2) forms a self-organizing network architecture by dynamically forming a virtual topology.

Virtual Topology Formation involves 3 steps in a cycle:

- Neighbour Finding: A node broadcasts a "REQ" (request) message and receives "RPLY" (reply) messages from neighbours.

- Best Neighbour Selection: The node calculates the connectivity duration (t_d) with each neighbour and selects the neighbour with maximum t_d.

- Set Duty Cycle: The selected node is set to 'Active' mode for t_d time, forming the virtual topology, and the process repeats.

Bangla:
Tic-tac-toe-arch হলো underwater sensor network এর জন্য একটা self-organizing virtual architecture, যেটা (১) underwater node গুলোর মধ্যে connectivity কতক্ষণ থাকবে সেটা calculate করে এবং (২) dynamically virtual topology তৈরি করে একটা self-organizing network architecture গড়ে তোলে।

Virtual Topology Formation এ একটা cycle এ 3টা step থাকে:

- Neighbour Finding: একটা node "REQ" (request) message broadcast করে এবং neighbour দের থেকে "RPLY" (reply) message পায়।

- Best Neighbour Selection: Node প্রতিটা neighbour এর সাথে connectivity duration (t_d) calculate করে এবং যার t_d সবচেয়ে বেশি সেই neighbour কে select করে।

- Set Duty Cycle: Selected node কে t_d সময়ের জন্য 'Active' mode এ রাখা হয়, এভাবে virtual topology তৈরি হয়, আর প্রসেসটা আবার repeat হয়।

Q9. What is Coverage in WSN? Explain the relationship between Coverage and Connectivity. (5 marks)

English:
Coverage refers to how well the sensing field is monitored/tracked by sensors. It determines (as per application-specific criteria) where to deploy/activate static sensors, or how to plan the trajectory of mobile sensors. Coverage algorithms may be centralized, distributed, or localized, and sensor deployment can be deterministic or random.

Relationship between Coverage and Connectivity: Let r_s = sensing range and r_t = transmission range. If transmission range ≥ 2 × sensing range, then coverage implies connectivity — meaning if the area is fully covered, the network will automatically remain connected too. Since most sensors satisfy this condition in practice, coverage becomes the main issue to solve (connectivity gets handled automatically).

Bangla:
Coverage বলতে বোঝায় sensor দিয়ে sensing field কতটা ভালোভাবে monitor/track করা হচ্ছে। এটা ঠিক করে (application-specific criteria অনুযায়ী) static sensor কোথায় deploy/activate করতে হবে, অথবা mobile sensor এর trajectory কীভাবে plan করতে হবে। Coverage algorithm centralized, distributed অথবা localized হতে পারে, আর sensor deployment deterministic বা random হতে পারে।

Coverage এবং Connectivity এর সম্পর্ক: ধরি r_s = sensing range এবং r_t = transmission range। যদি transmission range ≥ 2 × sensing range হয়, তাহলে coverage থাকলেই connectivity automatically থাকবে — অর্থাৎ area পুরোপুরি covered থাকলে network ও connected থাকবে। বাস্তবে বেশিরভাগ sensor এই condition satisfy করে, তাই মূল সমস্যা হয়ে দাঁড়ায় coverage (connectivity নিজে থেকেই maintain হয়ে যায়)।

Q10. Discuss the different types of Coverage problems in Static WSNs. (5 marks)

English:
Coverage problems in static WSNs are classified into three types:

- Area Coverage: The goal is to cover an entire monitoring area/region. Includes energy-efficient random coverage and connected random coverage. A network is "connected" if any active node can communicate with any other active node. Zhang and Hou proved that if communication range R_c ≥ 2 × sensing range R_s, coverage implies connectivity.

- Point Coverage: The objective is to cover a set of discrete points (not the whole area). This can be random point coverage (sensors distributed randomly so every point is covered by at least one sensor) or deterministic point coverage (done in a deterministic/planned manner).

- Barrier Coverage: Ensures detection of intruders crossing a barrier/boundary. Classified as k-barrier coverage — 1-barrier coverage (covered by at least 1 sensor), 2-barrier (at least 2 sensors), k-barrier (at least k sensors). It is also classified as weak coverage (any path crossing the barrier is detected) and strong coverage (every path is detected with overlapping sensor circles forming a continuous barrier).

Bangla:
Static WSN এ coverage problem কে তিন ভাগে ভাগ করা যায়:

- Area Coverage: পুরো monitoring area/region cover করাই লক্ষ্য। এতে energy-efficient random coverage এবং connected random coverage থাকে। কোনো network "connected" হবে যদি যেকোনো active node অন্য যেকোনো active node এর সাথে communicate করতে পারে। Zhang আর Hou প্রমাণ করেছেন যে communication range R_c ≥ 2 × sensing range R_s হলে coverage থাকলেই connectivity থাকে।

- Point Coverage: এখানে একটা area না, বরং কতগুলো discrete point cover করাই উদ্দেশ্য। এটা random point coverage (sensor গুলো randomly ছড়িয়ে দেয়া হয় যাতে প্রতিটা point অন্তত একটা sensor দ্বারা covered থাকে) অথবা deterministic point coverage (পরিকল্পিতভাবে করা হয়) হতে পারে।

- Barrier Coverage: এটা নিশ্চিত করে যে কোনো intruder barrier/boundary পার হলে সেটা detect হবে। একে k-barrier coverage হিসেবে ভাগ করা হয় — 1-barrier (কমপক্ষে 1টা sensor দিয়ে covered), 2-barrier (কমপক্ষে 2টা sensor), k-barrier (কমপক্ষে k সংখ্যক sensor)। এছাড়া এটাকে weak coverage (barrier পার হওয়ার যেকোনো একটা path detect হলেই হবে) এবং strong coverage (প্রতিটা path-ই detect হতে হবে, sensor circle গুলো overlap করে একটা continuous barrier তৈরি করে) হিসেবেও ভাগ করা যায়।

Q11. Explain the Optimal Geographical Density Control (OGDC) Algorithm. (5 marks)

English:
OGDC is a distributed, localized coverage-maintenance algorithm based on the idea that an area is completely covered if there are at least two intersecting disks and all crossings (intersection points between disk boundaries) are covered.

Optimality conditions for minimizing overlap: If nodes A and B are fixed, node C should be placed such that OR = OQ. If all three nodes can change positions, then OP = OR = OQ. If all nodes have the same sensing range r_s, the optimal distance between them is √3 × r_s.

Algorithm steps:

- A starting node volunteers with probability p, chooses a random back-off time; if it hears nothing from neighbors, it declares itself the starting node with a preferred direction.

- On receiving the starting node's message, each node computes its deviation from the desired (ideal) position based on distance and angle, chooses a random back-off time, and when it expires, sends a "power ON" message, then declares its own position/direction.

- This process continues until the entire area is covered; nodes already covered go to sleep mode to save energy, and this process repeats every round.

Bangla:
OGDC হলো একটা distributed, localized coverage-maintenance algorithm যেটা এই idea এর উপর ভিত্তি করে যে, একটা area সম্পূর্ণভাবে covered হয় যদি অন্তত দুইটা disk intersect করে এবং সব crossing (disk boundary এর intersection point) covered থাকে।

Overlap minimize করার optimality condition: যদি node A ও B fixed থাকে, তাহলে node C এমনভাবে বসাতে হবে যাতে OR = OQ হয়। যদি তিনটা node ই position পরিবর্তন করতে পারে, তাহলে OP = OR = OQ হতে হবে। সব node এর sensing range r_s একই হলে, তাদের মধ্যে optimal distance হয় √3 × r_s।

Algorithm এর ধাপ:

- একটা starting node p probability দিয়ে volunteer করে, random back-off time নেয়; কোনো neighbor থেকে কিছু না শুনলে সে নিজেকে starting node হিসেবে ঘোষণা করে preferred direction সহ।

- Starting node এর message পেলে, প্রতিটা node distance ও angle এর ভিত্তিতে desired (ideal) position থেকে deviation calculate করে, random back-off time নেয়, সময় শেষ হলে "power ON" message পাঠায়, এরপর নিজের position/direction ঘোষণা করে।

- পুরো area cover না হওয়া পর্যন্ত এই process চলতে থাকে; যেসব node ইতিমধ্যে covered তারা energy বাঁচাতে sleep mode এ চলে যায়, এবং এই প্রসেস প্রতি round-এ repeat হয়।